# Nepal Engineering College

(Affiliated to Pokhara University)

# Report on:

LAB 4: DFT and IDFT.

# Submitted By:	Submitted To:

Name: Aruna Thapa		Department Of: CRN: 021-311	Electrical and Electronics Date: 2082/09/21		 Engineering

# Objective

In this lab session, we write MATLAB programs to compute DFTs and IDFTs of discrete time signals, plot their spectra, compare results using built-in and custom functions, analyze performance, and implement circular convolution and FFT-based spectral analysis.

# Theory:

The Discrete Fourier Transform (DFT) is a mathematical technique used to analyze the frequency content of discrete-time signals. It transforms a time-domain signal into its frequency-domain representation. The DFT can be computed using the basic formula, matrix multiplication, or the Fast Fourier Transform (FFT) algorithm for efficient computation. Inverse DFT (IDFT) reconstructs the original time-domain signal from its frequency-domain representation. Additionally, the circular convolution of two sequences demonstrates the DFT's utility in systems analysis. Spectral analysis, such as FFT of a voice signal, provides insights into real-world signal characteristics.

## MATLAB Program to compute DFT of x[n] = {1,2,3,4,5,6,7,8}

% CRN: 021-311 - DFT using basic formula x = [1,2,3,4,5,6,7,8];

N = length(x); X = zeros(1,N); for k = 0:N-1

for n = 0:N-1

X(k+1) = X(k+1) + x(n+1)*exp(-1i*2*pi*k*n/N); end

end

disp('DFT X(k):'); disp(X);

## Command Window Output:

DFT X(k):

Columns 1 through 2

36.0000 + 0.0000i -4.0000 + 9.6569i

Columns 3 through 4

-4.0000 + 4.0000i -4.0000 + 1.6569i

Columns 5 through 6

-4.0000 - 0.0000i -4.0000 - 1.6569i

Columns 7 through 8

-4.0000 - 4.0000i -4.0000 - 9.6569i

## b. User-defined function for DFT

% CRN: 021-311 - User-defined function for DFT function y = dft(x)

N = length(x); y = zeros(1,N); for k = 0:N-1

for n = 0:N-1

y(k+1) = y(k+1) + x(n+1)*exp(-1i*2*pi*k*n/N); end

end end

## c. Plot real, imaginary, absolute and phase

% CRN: 021-311- Plot DFT Components x = [1,2,3,4,5,6,7,8];

X = dft(x);

k = 0:length(X)-1;

subplot(2,2,1); stem(k, real(X)); title('Real Part');

subplot(2,2,2); stem(k, imag(X)); title('Imaginary Part'); subplot(2,2,3); stem(k, abs(X)); title('Magnitude');

subplot(2,2,4); stem(k, angle(X)); title('Phase');

Figure 1: A set of subplots illustrating the real, imaginary, magnitude, and phase

components of the Discrete Fourier Transform (DFT) of the discrete-time signal x[n]={1,2,3,4,5,6,7,8} The subplots effectively demonstrate the spectral characteristics of the signal.

## DFT using matrix multiplication

% CRN: 021-311- DFT using WN matrix multiplication x = [1,2,3,4,5,6,7,8]; N = length(x); n = 0:N-1;

k = n';

WN = exp(-1i*2*pi/N); WN_matrix = WN .^ (k*n); X = WN_matrix * x.';

disp('DFT using WN matrix:'); disp(X);

## Command Window Output:

DFT using WN matrix: 36.0000 + 0.0000i

-4.0000 + 9.6569i

-4.0000 + 4.0000i

-4.0000 + 1.6569i

-4.0000 - 0.0000i

-4.0000 - 1.6569i

-4.0000 - 4.0000i

-4.0000 - 9.6569i

## DFT using built-in fft

% CRN: 021-311- DFT using fft x = [1,2,3,4,5,6,7,8];

X = fft(x);

disp('DFT using fft:'); disp(X);

Command Window Output:

DFT using fft:

Columns 1 through 2

36.0000 + 0.0000i -4.0000 + 9.6569i

Columns 3 through 4

-4.0000 + 4.0000i -4.0000 + 1.6569i

Columns 5 through 6

-4.0000 + 0.0000i -4.0000 - 1.6569i

Columns 7 through 8

-4.0000 - 4.0000i -4.0000 - 9.6569i

## Compare time taken by three methods

% CRN: 021-311- Time comparison x = [1,2,3,4,5,6,7,8];

tic

X1 = dft(x); t1 = toc;

tic

N = length(x);

n = 0:N-1; k = n';

WN = exp(-1i*2*pi/N); WN_matrix = WN .^ (k*n); X2 = WN_matrix * x.';

t2 = toc;

tic

X3 = fft(x); t3 = toc;

fprintf('Time (function): %f\nTime (matrix): %f\nTime (fft): %f\n', t1, t2, t3);

## Command Window Ouput:

Time (function): 0.001609

Time (matrix): 0.000401

Time (fft): 0.000072

## IDFT by all three methods

% CRN: 021-311- IDFT comparisons

% Using inverse matrix method X = fft([1,2,3,4,5,6,7,8]);

N = length(X);

n = 0:N-1; k = n';

WN = exp(1i*2*pi/N); WN_matrix = WN .^ (k*n);

x_reconstructed = (1/N) * (WN_matrix * X.');

disp('Reconstructed x[n] using IDFT matrix:'); disp(x_reconstructed);

% Using ifft x_ifft = ifft(X);

disp('Reconstructed x[n] using ifft:'); disp(x_ifft);

## Command Window Output:

Reconstructed x[n] using IDFT matrix: 1.0000

2.0000 + 0.0000i

3.0000 + 0.0000i

4.0000 + 0.0000i

5.0000 - 0.0000i

6.0000 - 0.0000i

7.0000 - 0.0000i

8.0000 - 0.0000i

Reconstructed x[n] using ifft:

1	2	3	4	5	6	7	8

## Circular convolution of unequal sequences

% CRN: 021-311- Circular Convolution Function function y = circular_conv(x1, x2)

N = max(length(x1), length(x2)); x1 = [x1, zeros(1, N - length(x1))];

x2 = [x2, zeros(1, N - length(x2))];

y = ifft(fft(x1) .* fft(x2)); end

## Check using cconv

% CRN: 021-311- Using cconv x1 = [1,2,3]; x2 = [4,5];

y = cconv(x1, x2, max(length(x1), length(x2))); disp('Circular convolution using cconv:'); disp(y);

## Command Window Output:

Circular convolution using cconv:

19	13	22

## Spectrum of 2-second voice signal

clc; clear;

% Read voice signal

[voice, fs] = audioread('voice.wav');

% Use single channel if stereo voice = voice(:,1);

% Take exactly 2 seconds voice = voice(1:2*fs);

% Time-domain plot

t = (0:length(voice)-1)/fs; figure;

plot(t, voice); xlabel('Time (seconds)'); ylabel('Amplitude');

title('2-Second Voice Signal'); grid on;

% Frequency spectrum N = length(voice);

Y = fft(voice);

f = (0:N-1)*(fs/N);

figure;

plot(f(1:N/2), abs(Y(1:N/2))/N); xlabel('Frequency (Hz)'); ylabel('Magnitude'); title('Spectrum of Voice Signal'); grid on;

Figure 2: A plot representing the amplitude spectrum of a 2-second voice signal. The frequency axis spans both positive and negative frequencies, and the plot reveals the signal's dominant frequency components.

# Conclusion

In this lab, we successfully implemented various techniques to compute and analyze the Discrete Fourier Transform (DFT) and Inverse DFT (IDFT) of discrete-time signals using both user-defined functions and built-in MATLAB functions. We compared execution performance, visualized signal characteristics through magnitude and phase spectra, and implemented circular convolution. Additionally, we analyzed the frequency components of a real-world voice signal using FFT. These exercises enhanced our understanding of spectral analysis and its practical applications in digital signal processing.